PRENSADOS Y GALVANIZADOS DE LARA, S.A.

Extracted Data Report

Barquisimeto, Lara State, 3001, Venezuela  |  +584245715349  |  @pyglarasa

# 1. Company Overview

100% Venezuelan company and leader in hot-dip galvanizing services. Guarantees superior protection against rust and abrasion, extending the lifespan of metal structures and reducing maintenance costs.

Address: 26th Street, between 1st and 2nd Avenue, Warehouse No. 25-90, Barquisimeto, Lara State, 3001, Venezuela.

## 1.1 Hot-Dip Galvanizing Process

Raw Material: Zinc (Zn) — Special High Grade (99.9%) ingots, melted at 450°C to form alloy layers.

Chemical Inputs:

- Pickling Agents (Hydrochloric Acid / HCl): Removes surface rust and mill scale.

- Flux (Zinc Ammonium Chloride): Removes last traces of oxide, prevents re-oxidizing, facilitates molten zinc wetting.

# 2. Specialized Infrastructure (Kettle Dimensions)

| Kettle Type | Dimensions | Application |
| --- | --- | --- |
| Large-Format Pilling Kettle | 7.00 x 0.80 x 1.40 m | Long profiles & large-scale structures |
| Mid-Range Kettle | 3.00 x 0.60 x 1.00 m | Intermediate parts & industrial hardware |
| Precision Kettle | 65 x 65 x 65 cm | Small parts & fasteners |

Installed production capacity: 400 tons per month.

# 3. 9-Meter Machinery for Installation

## 3.1 Major Equipment Costs (USD)

| Description | Qty | Unit Price ($) | Total ($) |
| --- | --- | --- | --- |
| Structure for Galvanizing Furnace | 1 | 285,000 | 285,000 |
| Refractory Ceramics, Mineral Wool & Anchors | — | 65,000 | 65,000 |
| Kettle/Caldron (9m x 80cm x 1.40m, 2" thick) | 1 | 80,000 | 80,000 |
| Fabricated Metal Structure | 1 | 100,000 | 100,000 |
| Plans & Assembly Consultancy | 1 | 40,000 | 40,000 |
| TOTAL |  |  | 570,000 |

## 3.2 Key Combustion Components

4x Hauck RSV120 burners, 4x Kromschröder pulse-fired butterfly valves, 4x Apollo gas ball valves, 4x North American air/gas ratio regulators, 4x Hauck orifice meters, 4x Hauck orifice limiting valves, Spencer air combustion fan (375 CFM @ 16 OSI, 5hp TEFC motor), Spencer fan inlet filter, Allen Bradley fan starter equipment, 3x air and gas pressure gauges, 1x 1-1/2" RP&C main gas filter, 2x 1-1/2" Apollo main gas shut-off valves, 1x 1" Equimeter main gas pressure regulator, 3x air and gas pressure switches, 2x 1-1/2" Honeywell gas shut-off safety valves, 1x 1-1/2" Honeywell gas vent valve, 12x 1/8" Conbraco test cocks, 4x 3/4" Kromschröder gas burner solenoid valves, 4x Honeywell ignition transformers, ignition cable set with connectors, 4x Hauck burner bases, 2x Type J thermocouples for kettle, 2x weldable support bushings for kettle, 2x copper plugs, 1x oval ceramic fiber tube, 1x Type K thermocouple for chimney/flue.

## 3.3 Control Panel Board (30" x 36" x 16")

1x Honeywell DC330 temperature controller, 1x Honeywell DC230 chimney temperature controller, 1x Honeywell DC230L bath limit temperature controller, 1x Monarch 2-channel paperless recorder, 1x Kromschröder PF 19 pulse/flame safety rack, 1x Eagle signal timer, 1x Edwards main control furnace alarm, 1x Allen Bradley selector switch, 3x Allen Bradley push buttons, 8x Allen Bradley pilot lights, 1x Allen Bradley MicroLogix PLC, lot of terminal blocks/fuses.

## 3.4 Refractory Materials

1,200 sq ft of 1" thick ceramic fiber (#8 density), 600 sq ft of 2" thick ceramic fiber (#6 density), 1,200 sq ft of 2" thick mineral wool blocks, 800x 310 SS 8" stud sets with locking washers, 300x 310 SS 6" stud sets, 100x 310 SS 4" stud sets, 140x 310 SS 2" stud sets, 100 lbs of 310 SS mesh, 900 lbs of insulating mortar.

# 4. Installed Production Capacity

## 4.1 Raw Material Requirements

| Material | Quantity | Price (USD) | Total (USD) |
| --- | --- | --- | --- |
| Zinc SHG 99.9% | 140 Tons / Year | $3,600.00 | $504,000.00 |
| Hydrochloric Acid | 5 Tons / Month | $1,173.91 | $5,869.56 |
| Ammonium Chloride | 1 Ton / Month | $2,300.00 | $2,300.00 |

## 4.2 Galvanizing Yield with 140 Tons of Zinc

- High-efficiency scenario (7%): Up to 2,000 tons of steel (large/heavy parts, structural beams)

- Standard scenario (10%): 1,400 tons of steel (medium profiles, pipes, general ironwork)

## 4.3 Fixed Monthly Expenses

| Item | Amount / Month |
| --- | --- |
| Gas | $1,800 |
| Electricity | $200 |
| Security | $1,200 |
| Salaries / Wages | $2,300 |
| Cleaning / Maintenance | $100 |
| TOTAL | $5,600 |

# 5. Copper Electrolytic Plant

Manufactures electrical grounding rods (5/8" to 1" diameter, 1.20m to 3m length). Copper layer thickness: 300 µm ± 50. Installed capacity: 936 rods/day on continuous 24-hour cycle.

## 5.1 Chemical Inputs Required

| Item (Spanish) | English | Quantity / Details |
| --- | --- | --- |
| Ácido | Acid | 5,000 kg / month |
| Cloruro de Amonio | Ammonium Chloride | 1,000 kg / month (startup) |
| Planta de Cobre | Copper Plant | 4 tons electrolytic copper (4 tanks) |
| Sulfato de Níquel | Nickel Sulfate | 600 kg |
| Cloruro de Níquel | Nickel Chloride | 120 kg |
| Ácido Bórico | Boric Acid | 100 kg |
| Níquel en Briquetas | Nickel Briquettes | 1,000 kg |
| Cesta de Titanio | Titanium Basket | — |
| Baños de Cobre | Copper Baths | 3,600 Copper Sulfate bath |
| Ácido Sulfúrico | Sulfuric Acid | 900 |

# 6. Equipment Reference

Kettles manufactured by W. Pilling Riepe GmbH & Co. KG, Gutenbergstrasse 2, 38652 Halon, Germany. Equipment shown includes new and in-service kettles, chemical treatment tanks with transformers, pickling/treatment line, small galvanizing kettle with perforated basket, and the large galvanizing furnace structure.